Er Manish Bhadoria’s Interactions Study Circle 10th CBSE Science

21. In the circuit diagram given here, the

current flowing in 5 ohm resistor is 1

ampere. Find the current flowing through

the other two resistors.

22. How will you connect three resistors of 2 W, 3 W and 5 W respectively so as

to obtain a resultant resistance of 2.5 W? Draw the diagram to show the

arrangement.

23. Suppose a 6-volt battery is connected

across a lamp, whose resistance is 20

ohm, through a variable resistor as

shown in fig. If the current in the

circuit is 0.25 A, calculate the value of

the resistance from the resistor which

must be used.

24. In the circuit diagram given in figure,

calculate:

(i) The current through each resistor,

(ii) The total current in the circuit, and

(iii) The total circuit resistance.

25. If you connect three resistors having values 2 W, 3 W and 5 W in parallel,

then, will the value of the total resistance be less than 2 W, or greater than 5

W, or lie between 2 W and 5 W? Explain.

26. Five dry cells each of 1.5 volt have internal resistance of 0.2, 0.3, 0.4, 0.5

and 12 ohms. When connected in series, what current will these five cells

furnish through 10 ohm resistance?

Er Manish Bhadoria’s Interactions Study Circle 10th CBSE Science

12 Electricity

(Problems Sheet – 2)

Problems on Combination of Resistances

1. If four resistances, each of value 1 ohm, are connected in series, what will

be the resultant resistance?

2. A resistance of 6 ohms is connected in series with another resistance of 4

ohms. A potential difference of 20 volts is applied across the combination.

Calculate the current through the circuit and potential difference across

the 6 ohm resistance.

3. Calculate the equivalent resistance when two resistances of 3 ohms and 6

ohms are connected in parallel.

4. How should the two resistances of 2 ohms each be connected so as to

produce an equivalent resistance of 1 ohm?

5. Three resistances of 12, 15 and 20 ohms are connected first in series and

then in parallel. What is the equivalent (or combined) resistance in each

case?

6. How can three resistors of 2, 3 and 6 ohms be connected in a circuit to give

a total resistance of 1 ohm?

7. What possible values of resistance are get by combining two resistance,

one of value 2 W and other 6 W?

8. If three resistances of 3 ohms each are connected in parallel, what will be

their total resistance?

9. In the circuit diagram, find:

(i) Total resistance

(ii) Current shown by the ammeter A.

Er Manish Bhadoria’s Interactions Study Circle 10th CBSE Science

10. Two resistances when connected in parallel give resultant value of 2 ohm.

When they are connected in series, the value becomes 9 ohms. Calculate

the value of each resistance.

11. A resistor of 8 ohms is connected in parallel with another resistor X. the

resultant resistance of the combination is 4.8 ohms. What is the value of

the resistor X?

12. Two resistances are connected in

series as shown in the fig.

(i) What is the current through the 5

ohm resistance?

(ii) What is the current through R?

(iii) What is the value of R?

(iv) What is the value of V?

13. In the circuit diagram given here,

find:

(i) Total resistance of the circuit,

(ii) Total current flowing in the

circuit, and

(iii) The potential difference across

7.2 W resistance.

14. In the diagram, the cell and the

ammeter both have negligible

resistance. The resistors are

identical. With the switch K open,

the ammeter reads 0.6 A. What will

be the ammeter reading when the

switch is closed?

15. Three resistors are connected as shown in the diagram. Through the

resistor 5 ohm, a current of 1 ampere is flowing.

Er Manish Bhadoria’s Interactions Study Circle 10th CBSE Science

(i) What is the current through

other two resistors?

(ii) What is the p.d. across AB

and across AC?

(iii) What is the total resistance?

16. For the circuit shown in the following

diagram, find the value of:

(i) Current through 6 W resistor.

(ii) Potential difference across 12 W resistor.

17. Two resistors X and Y are connected turn by turn: (i) in parallel, and (ii) in

series. In which case the resultant resistance will be less than either of the

individual resistances?

18. You are given three resistances of 1, 2 and 3 ohms. Show by diagrams,

how with the help of these resistances you can get:

(i) 6 W, (ii) 6/11 W, (iii) 1.5 W, (iv) 4/3 W, (v) 5/6 W, (vi) 11/5 W, (vii) 11/4 W,

(viii) 11/3 W

19. Four resistances of equal value 1 ohm are given. Show by diagrams all

different variations in which these four resistances can be connected.

Calculate in each case the resultant resistance of the combination. In which

case is the total resistance highest and in which it is lowest?

20. The circuit diagram here shows the

combination of three resistors. Find:

(i) Total resistance of the circuit.

(ii) Total current flowing in the circuit.

(iii) Potential difference across 4 W

resistor.

Er Manish Bhadoria’s Interactions Study Circle 10th CBSE Science

27. Calculate the effective resistance

between the points A and B in the

network shown here.

28. Calculate the effective resistance in the

diagram shown in figure.

29. In the circuit given find the current

through each resistor.

30. When two resistances are connected in series, their combined resistance is

25 ohms and when they are connected in parallel, their combined

resistance is 4 ohms. Find their individual resistance.

31. In the circuit shown in figure, calculate

(i) The current flowing through the

arms AB, AC and CDE.

(ii) The potential difference across AB,

CD and DE.

Er Manish Bhadoria’s Interactions Study Circle 10th CBSE Science

32. Four resistances of 16 ohms each are connected in parallel. Four such

combinations are connected in series. What is the total resistance?

33. Six equal resistors of 1 W each are connected to form the sides of a hexagon

ABCDEFG. Calculate the resistance offered by the combination if the

current:

(i) Enters at A and leaves at B.

(ii) Enters at A and leaves at C.

(iii) Enters at A and leaves at D.

34. Calculate the equivalent

resistance of the combination

of resistances shown in

figure.

35. Calculate the effective

resistance between the points

A and B in the circuit shown

in figure.

36. In the circuit shown

here, calculate the

equivalent resistance

between the points A

and B.

37. A circuit is set up as shown in

figure. Calculate the current

through and the potential

difference across R1, R2 and R3,

when:

(i) Keys K1 and K2 both are closed.

(ii) Key K1 is closed and K2 is opened.

(iii) Key K1 is opened and K2 is closed.   ---------- x ----------